# Floorwatch — CCTV Integration & Build Pipeline

Technical plan for moving from the demo prototype to a live system running on real camera feeds

Audience: engineering/technical stakeholders and IT contacts on the Cineplex side. Companion to the plain-language one-pager, which is written for non-technical decision-makers.

# 1. What changes vs. the demo

The prototype dashboard (floor status, event feed, supervisor queue, task effort cards) is already functionally complete — it renders whatever events it receives. In the demo, those events come from a scripted sequence the presenter clicks through. In the real system, they come from live camera feeds through a detection pipeline. The dashboard code does not need to be rebuilt; a real event source needs to be built and connected to it.

# 2. End-to-end pipeline

| 1 | Camera access Read-only connection to existing CCTV feeds via RTSP or ONVIF, negotiated with the Cineplex's NVR/DVR system. No new cameras required for the pilot. |
| --- | --- |

| 2 | Edge ingestion An on-prem edge box at the cineplex pulls the video stream(s), decodes frames at a sampled rate (e.g. 2–5 fps is usually enough — full frame-rate is unnecessary and wastes compute). |
| --- | --- |

| 3 | Detection models Two model passes per frame: (a) person detection + pose/motion classification for coverage and active-time signals (Part B and Part A base layer); (b) optional role/uniform tagging if role-based tagging is desired over manual roster mapping. |
| --- | --- |

| 4 | Zone & task mapping Detected person coordinates are mapped to calibrated zone polygons (drawn once per camera during setup). For Part A, active-time is accumulated per open task-assignment window, not just per zone. |
| --- | --- |

| 5 | Event generation The mapping layer emits structured events matching the schema the dashboard already expects — zone_covered, zone_gap, task_active_time_update, task_flag, etc. — onto an internal event bus. |
| --- | --- |

| 6 | Rules engine Consumes events, applies debounce/confidence thresholds, cooldowns, and the escalation state machine (tier 1 nudge → tier 2 supervisor command → tier 3 logged) exactly as designed in the earlier report — this logic is already validated in the prototype and ports over unchanged. |
| --- | --- |

| 7 | Delivery layer Structured, non-video messages leave the edge boundary here: push notifications to employee devices, supervisor directive cards, and the live dashboard feed via WebSocket — the same dashboard already built. |
| --- | --- |

| 8 | Storage & reporting Structured events (not video) are stored per the Cineplex's retention policy, feeding the shift digest and pattern reports. Raw video stays in the existing CCTV/DVR system and is never duplicated into a second store. |
| --- | --- |

# 3. Exactly what replaces the simulator in the dashboard code

The prototype's script[] array and its manual run() calls are removed. In their place:

const ws = new WebSocket("wss://<edge-or-backend>/events");
ws.onmessage = (msg) => {
  const evt = JSON.parse(msg.data);
  applyEvent(evt); // same function shape as the demo’s state updates
};

The applyEvent() function takes over the role the scripted steps used to play — reading action_type off a real event and updating zoneState / tasks accordingly. Every rendering function (renderZones, renderQueue, renderTasks, renderEmpPreview) is reused without modification, since they were built against the event schema, not the simulator.

# 4. Event schema (shared by Part A and Part B)

{
  "event_id": "uuid",
  "timestamp": "ISO8601",
  "camera_id": "string",
  "zone_id": "string",
  "role_tag": "concession | usher | janitor | maintenance | ticketing | security",
  "entity_ref": "anonymous_track_id (default) | employee_id (only if escalated)",
  "event_type": "zone_covered | zone_gap | task_assigned | task_active_time_update | task_flag | task_resolved",
  "task_id": "string | null",
  "active_minutes": "number | null",
  "assigned_minutes": "number | null",
  "confidence": "0.0-1.0",
  "source_model_version": "string"
}

task_id, active_minutes, and assigned_minutes are the new fields added for Part A; everything else already existed for Part B in the earlier design. Keeping one shared schema is what let the dashboard support both without a rebuild.

# 5. Build phases after CCTV access is granted

## Phase 1 — Connect and calibrate (1–2 weeks)

- Confirm RTSP/ONVIF credentials and stream stability for the 1–2 pilot cameras.

- Stand up the edge box on-site; validate frame ingestion and latency.

- Run the zone-calibration tool: draw zone polygons over each camera's frame, label with role/zone metadata.

- Validate detection accuracy against a few hours of manual spot-checking before anything reaches an employee.

## Phase 2 — Wire Part B (coverage) to real data (1–2 weeks)

- Connect zone-mapping output to the rules engine in shadow mode: log gap events, do not send nudges yet.

- Compare shadow-mode gap detections against manager observation for a set period to tune the debounce/confidence thresholds.

- Turn on real nudges and supervisor commands once shadow-mode accuracy is acceptable.

## Phase 3 — Wire Part A (effort tracking) to real data (2–3 weeks)

- Build the task-assignment input (a simple form or integration with an existing scheduling tool) so supervisors can log zone + time budget per task.

- Connect active-time accumulation per task window using the same motion/pose signal already running for Part B.

- Run in shadow mode first here as well — this is the more sensitive capability, and false flags are more damaging than false coverage gaps, so extra validation time is worth it before nudges/flags go live.

- Calibrate what “normal” active-time ratios look like per task type before setting flag thresholds — avoid a single hardcoded percentage across very different task types.

## Phase 4 — Full pilot rollout (2–4 weeks)

- Expand from 1–2 cameras to the full pilot zone set agreed with the Cineplex.

- Turn on the supervisor dashboard and employee-facing notifications for real use.

- Begin collecting real shift-digest data for the pilot review checkpoint.

# 6. Infrastructure requirements

| Component | Requirement | Notes |
| --- | --- | --- |
| Edge compute | On-prem GPU-capable box per site (e.g. Jetson-class or small server) | Keeps raw video processing on-site, not in the cloud |
| Network | Read-only access to camera stream(s) via RTSP/ONVIF | No write access to the Cineplex's CCTV/NVR system needed |
| Event transport | Lightweight message bus (e.g. Redis Streams / MQTT) edge-to-cloud | Only structured events cross this boundary, never video |
| Storage | Structured event store (time-series or relational), Cineplex-defined retention | No new video storage — raw footage stays in the existing DVR |
| Dashboard hosting | Existing prototype's frontend, pointed at the real WebSocket endpoint | No rebuild required, per Section 3 |

# 7. Validation before go-live

- Detection accuracy spot-check: manually verify a sample of gap and effort-flag events against what actually happened, before nudges/flags are shown to real employees.

- Bias check across shifts/lighting/camera angles — accuracy shouldn't vary meaningfully by time of day or which employee is on shift.

- Roster cross-check wired in before go-live, so an unstaffed zone never generates a false gap or false effort flag.

- Sign-off from the Cineplex on staff notice/consent status — real data should not start flowing until this is confirmed, per the responsibility split in the earlier report.

# 8. What does not change from the original architecture report

- Responsibility split (Section 2 of the earlier report) — unchanged.

- Escalation state machine, cooldowns, and thresholds — unchanged, just fed by real events instead of a script.

- Default to anonymous, zone-level detection rather than persistent employee identity — unchanged, including for Part A effort tracking.